Says on the Silicon board 2004A

I2C bus is a type of serial bus invented by PHLIPS.

It is a high-performance serial bus which has bus ruling and high or low speed device synchronization function required by multiple host system.

I2C bus has only two bidirectional signal lines:

- Serial Data Line (SDA)

- Serial Clock Line (SCL)

Contains blue potentiometer:

- The I2C LCD2004 is used to adjust the contrast (backlight) to make it easier to display on the I2C LCD2004.

- Increased:		Clockwise

- Decreased:		Counter-clockwise

- Backlight/Contrast

Jumper Cap

- Enable Backlight:		Jumper cap connected

- Disable backlight:	Jumper cap disconnected (Unplugged)

- I2C Address

- Default address:			0x27

- Addresses for some case:	0x3F

- Address Modification:		By shorting the address pads (A0, A1, A2)

  - Default:			A0/A1/A2 is 1

  - Shorted:			A0/A1/A2 is 0

| - Pin Connections |  |  |
| --- | --- | --- |
| - PIN | - Description | - Connection |
| - GND | - Ground |  |
| - VCC | - Voltage supply | - 5V |
| - SDA | - Serial data line | - Connect to VCC through a pull_up resistor |
| - SCL | - Serial clock line | - Connect to VCC through a pull_up resistor |